7I77 SIX AXIS ANALOG SERVO PLUS I/O DAUGHTERCARD

V1.6

CAUTION!

The 7I77s intended application is interfacing large servo motors. Servo motors are capable of inflicting serious injury both to people and mechanisms associated with the servo system. In addition, some motors use potentially lethal supply voltages.

When a servo system is first configured, unpredictable behavior should be EXPECTED. First time checks of basic servo operation (such as motor position versus analog output drive) should be done with the motor power leads disconnected.

NEVER depend on software commands to disable a motor when you or others would be exposed to a hazard should the motor start unexpectedly. Motor power should be always be removed when working on mechanical parts of the servo system.

Be especially careful with encoder wiring, as a simple bad connection of one encoder wire can lead to total loss of control and a runaway servo system.

GENERAL	 1

DESCRIPTION	 1

HARDWARE CONFIGURATION 	2

GENERAL 	 2

VIN POWER SOURCE	2

CABLE 5V POWER	2

SETUP/OPERATE MODE 	 2

ENCODER INPUT MODES 	3

CONNECTORS 	 4

7I77 CONNECTOR LOCATIONS AND DEFAULT JUMPER POSITIONS .... 4

P1 HOST INTERFACE	5

TB1AUX 5V CONNECTOR 	6

TB2 FIELD POWER CONNECTOR 	 6

TB3 ENCODER 0 THROUGH 2 	 7

TB4 ENCODER 3 THROUGH 5 	 8

TB5 ANALOG DRIVE CONNECTOR 	 9

TB6 RS-422 INTERFACE	 10

FIELD INPUT/OUTPUT CONNECTORS 	 11

TB8 PINOUT 	 11

TB7 PINOUT 	 12

OPERATION 	 13

HOST INTERFACE 	 13

STATUS LEDS 	 13

EXTERNAL 5V POWER	 13

ANALOG SERVO DRIVE INTERFACE 	 14

SERVO ENABLE OUTPUTS 	 14

ANALOG SECTION WATCHDOG 	 14

ENCODER INTERFACE 	 14

MAXIMUM ENCODER COUNT RATE 	 14

RS-422 INTERFACE 	 15

OPERATION

FIELD I/O	 16

FIELD AND VIN POWER SUPPLY 	 16

FIELD OUTPUT CHARACTERISTICS	 16

WHY SOURCING OUTPUTS 	 16

SHORT CIRCUIT PROTECTION	 16

OVER-TEMPERATURE PROTECTION 	 16

MAXIMUM PER CHIP CURRENT 	 16

VOLTAGE CLAMPS 	 17

FIELD INPUT CHARACTERISTICS	 17

WHY SINKING INPUTS	 17

ANALOG INPUTS 	 17

FIELD VOLTAGE MONITORING 	 17

MPG INPUTS 	 18

WATCHDOG AND FAULTS 	 18

FIELD I/O PARAMETERS 	 18

NON-VOLATILE FIELD I/O PARAMETERS	 19

OPERATE MODE BAUD RATE	 19

WATCHDOG TIMEOUT	 19

RPD, WPD, AND UFLBP	20

SOFTWARE PROCESS DATA MODES 	21

REFERENCE INFORMATION

SSLBP 	22

GENERAL	22

REGISTER MAP 	22

PROCESS INTERFACE REGISTERS	22

COMMAND REGISTER	23

COMMAND REGISTER WRITE IGNORE	23

DATA REGISTER 	24

LOCAL READ OPERATIONS 	24

LOCAL WRITE OPERATIONS 	24

LOCAL PARAMETERS 	25

NORMAL START	26

STOP ALL	27

STOP INDIVIDUAL CHANNELS 	27

DOIT 	27

PER CHANNEL INTERFACE DATA REGISTERS	28

PER CHANNEL CONTROL AND STATUS REGISTERS 	28

REMOTE MODES	28

INTERFACE AND CS REGISTER CONTENTS AT START	28

CS REGISTER AFTER START	30

CS REGISTER AFTER DOIT	30

PROCESS DATA DISCOVERY	31

PROCESS TABLE OF CONTENTS	32

PROCESS DATA DESCRIPTOR	32

PROCESS DATA DESCRIPTOR FIELDS	32

RECORD_TYPE 	32

DATA_LENGTH	32

DATA_TYPE 	33

DATA_DIRECTION 	33

PARAM ETE R_M IN 	33

PARAM ETE R_MAX	33

UNIT_STRING	34

NAME_STRING	34

NUMERIC PROCESS DATA SCALING	34

MODE DESCRIPTOR 	34

MODE TYPES	34

PROCESS ELEMENT PACKING AND UNPACKING 	35

7I77 SPECIFIC PROCESS DATA EXAMPLE 	37

NORMAL MODE OPERATION 	38

SETUP START 	39

SETUP MODE OPERATION	39

REMOTE READ EXAMPLE	39

REMOTE WRITE EXAMPLE	40

DISCOVERY SEQUENCE	41

REFERENCE INFORMATION 	44

LBP 	44

LBP DATA READ/WRITEWCOMMAND 	44

EXAMPLE COMMANDS 	45

LOCAL LBP COMMANDS	46

LOCAL LBP READ COMMANDS	46

LOCAL LBP WRITE COMMANDS	48

RPC COMMANDS	48

EXAMPLE RPC COMMAND LIST	50

CRC	51

FRAMING 	51

SPECIFICATIONS 	52

DRAWINGS	54

GENERAL

DESCRIPTION

The 7I77 is daughtercard/breakout board for use with MESA's 25 pin I/O FPGA cards like the 5I25. The 7I77 is designed for interfacing up to 6 Axis of analog servo motor drives and also provides 48 isolated I/O points for general purpose field I/O use.

Six +-10V analog outputs and six encoder inputs (all with index) are provided for servo interfacing. Six floating drive enable outputs allow control of active high or active low drive enables. Encoder inputs can be individually set for differential or single ended mode.

48 points of isolated field I/O are provided for general control use including limit switch, control panel inputs, coolant enable and tool changer control outputs. Isolated I/O includes 32 sinking inputs and 16 sourcing outputs. Inputs can sense 5V to 32V signals and the outputs can switch 5V through 28V signals. Maximum output load is 300 mA. Outputs are short circuit protected. Field I/O is powered by an isolated 10-32V field power source.

One RS-422 interface is provided for I/O expansion via a serial I/O daughtercard. All field wiring is terminated in pluggable 3.5 mm screw terminal blocks.

GENERAL

Hardware setup jumper positions assume that the 7I77 card is oriented in an upright position, that is, with the host interface DB25 connector pointing towards the left.

VIN POWER SOURCE

The isolated field I/O on the 7I77 runs from a switching power supply that can be powered by field power or a separate supply (VIN) with ground common with field power. Normally the 7I77 field I/O logic will be powered with field power and an on card jumper, W1 allows VIN to be connected to field power. If you wish to use a single power supply for the 7I77s output and logic power, W1 should be placed in the left hand position. This connects field power to VIN. If you wish to use a separate supply for VIN, W1 Should be placed in the right hand position.

CABLE 5V POWER

The 7I77 can get its logic power from the host FPGA card or from TB1. W5 determines if the 7I77 gets its 5V logic power from the host FPGA card or TB1.

If W5 is in the left hand position, host FPGA power is used and the host FPGA card must be jumpered to supply 5V to the daughtercard.

If W5 is in the right hand position, 5V power must be supplied to the 7I77 via TB1 and the 7I77 grounds the 4 DB25 signals used for host 5V power. In this case the FPGA card must be jumpered so that it does not supply power to the daughtercard.

Never apply external power to the 7177 when W5 Is in the left hand position or you may damage the 7177, FPGA card, PC, or connecting cable.

SETUP/OPERATE MODE

The 7I77 field I/O and Analog output processors can run in setup mode or operate mode. In setup mode, the serial interface baud rate is fixed at 115.2 KBaud. In the operate mode, the baud rate is set to 2.5M baud (default). Setup mode is also less critical of host interface timing and enables a normal PC to communicate with the 7I77 for setup and firmware update purposes. W4 controls the setup/operate mode selection for the Analog processor and W12 controls the setup/operate mode selection for the isolated field I/O processor. The jumpers must be in the "operate" mode for normal operation.

W4,W12	MODE	BAUD RATE

LEFT	Operate mode	2.5M baud (default, can be changed)

RIGHT	Setup Mode	115.2K baud (fixed)

ENCODER INPUT MODES

The 7I77s six encoder inputs can be programmed for differential or single ended mode operation. Each encoder has 3 jumpers which set the input mode of the individual A/B/Z inputs. Normally these jumper will all be set to single ended or differential on a single encoder channel. The jumpers are set to the right hand position for differential inputs and the left hand position for single ended inputs. Note that the groups of three jumpers are close to the associated encoder connector. Default setting is differential (right hand position) for all encoders.

| ENCODER | A | B | Z |
| --- | --- | --- | --- |
| 0 | W21 | W19 | W17 |
| 1 | W15 | W13 | W10 |
| 2 | W8 | W6 | W2 |
| 3 | W22 | W20 | W18 |
| 4 | W16 | W14 | W11 |
| 5 | W9 | W7 | W3 |

P1 HOST INTERFACE CONNECTOR

P1 is the DB25F connector on the 7I77 that connects to the FPGA card. Actual pin functions depend on FPGA configuration but signal directions must be observed.

| DB25 PIN | GPIO | FUNCT | DIR | DB25 PIN | GPIO | FUNC | DIR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | IO0 | SS2TXEN | OUT | 14 | IO1 | SS2TX | OUT |
| 2 | IO2 | SS2RX | IN | 15 | IO3 | SS1TX | OUT |
| 3 | IO4 | SS1RX | IN | 16 | IO5 | SS0TX | OUT |
| 4 | IO6 | SS0RX | IN | 17 | IO7 | ENCMUX | OUT |
| 5 | IO8 | QCMUXA01 | IN | 18 | GND |  |  |
| 6 | IO9 | QCMUXB01 | IN | 19 | GND |  |  |
| 7 | IO10 | QCMUXI01 | IN | 20 | GND |  |  |
| 8 | IO11 | QCMUXA23 | IN | 21 | GND |  |  |
| 9 | IO12 | QCMUXB23 | IN | 22 | GND | or 5V |  |
| 10 | IO13 | QCMUXI23 | IN | 23 | GND or 5V |  |  |
| 11 | IO14 | QCMUXA45 | IN | 24 | GND or 5V |  |  |
| 12 | IO15 | QCMUXB45 | IN | 25 | GND or 5V |  |  |
| 13 | IO16 | QCMUXI45 | IN |  |  |  |  |

Notes:

- If jumper W5 is is the left hand position, pins 22 through 25 are 5V, if W5 is in the right hand position, Pins 22 through 25 are GND.

- GPIO pins are for first FPGA connector, next connector series begins at GPIO17.

- Signal directions are relative to FPGA card, that is, an ‘OUT’ signal is an output from the FPGA card that drives the 7I77. Conversely an ‘IN’ signal is a FPGA input that is driven by the 7I77.

TB1 AUX 5V CONNECTOR

TB1 supplies 5V analog, encoder, and RS-422 interface power to the 7I77. If the cable power option is used, TB1 may remain unconnected. If the cable power option is not used TB1 must be used to supply 5V power to the 7I77.

When TB1 is used with an external 5V supply (not 5V from the host PC) , W5 must be in the right-hand position to avoid back-powering the host computer via the host interface card. TB1 is a two pin 3.5 MM pluggable terminal block supplied one two pin removable screw terminal plug. TB1 pinout is as follows:

TB1 PIN SIGNAL

- GND

- 5V

TB2 FIELD POWER

TB2 is the 7I77s field power connector. TB2 is a eight pin 3.5 MM pluggable terminal block supplied one eight pin removable screw terminal plugg. TB2 pinout is as follows:

| TB2 PIN | SIGNAL | FUNCTION |
| --- | --- | --- |
| 1 | VFIELD | FIELD POWER 8-32V (Bottom pin) |
| 2 | VFIELD | FIELD POWER 8-32V |
| 3 | VFIELD | FIELD POWER 8-32V |
| 4 | VFIELD | FIELD POWER 8-32V |
| 5 | VIN | FIELD I/O LOGIC POWER 8-32V |
| 6 | NC |  |
| 7 | NC |  |
| 8 | GROUND | VIN, VFIELD, COMMON (Top pin) |

Note: When W1 is in the default left hand position, VIN is connected to VFIELD, so only VFIELD need be supplied to the 7I77 to power its field IO.

TB3 ENCODER 0 THROUGH 2

TB3 is the 7I77s encoder 0 through 2 connector. Each encoder interface uses 8 pins TB3 is a 24 pin 3.5 MM pluggable terminal block with supplied with three eight pin removable screw terminal plugs.

| TB3 PIN | FUNCTION | DIR |
| --- | --- | --- |
| 1 | QA0 | TO 7I77 |
| 2 | /QA0 | TO 7I77 |
| 3 | GND | FROM 7I77 |
| 4 | QB0 | TO 7I77 |
| 5 | /QB0 | TO 7I77 |
| 6 | +5V | FROM 7I77 |
| 7 | IDX0 | TO 7I77 |
| 8 | /IDX0 | TO 7I77 |
| 9 | QA1 | TO 7I77 |
| 10 | /QA1 | TO 7I77 |
| 11 | GND | FROM 7I77 |
| 12 | QB1 | TO 7I77 |
| 13 | /QB1 | TO 7I77 |
| 14 | +5V | FROM 7I77 |
| 15 | IDX1 | TO 7I77 |
| 16 | /IDX1 | TO 7I77 |
| 17 | QA2 | TO 7I77 |
| 18 | /QA2 | TO 7I77 |
| 19 | GND | FROM 7I77 |
| 20 | QB2 | TO 7I77 |
| 21 | /QB2 | TO 7I77 |
| 22 | +5V | FROM 7I77 |
| 23 | IDX2 | TO 7I77 |
| 24 | /IDX2 | TO 7I77 |

TB4 ENCODER 3 THROUGH 5

TB4 is the 7I77s encoder 3 through 5 connector. Each encoder interface uses 8 pins. TB4 is a 24 pin 3.5 MM pluggable terminal block supplied with three eight pin removable screw terminal plugs.

| TB4 PIN | FUNCTION | DIR |
| --- | --- | --- |
| 1 | QA3 | TO 7I77 |
| 2 | /QA3 | TO 7I77 |
| 3 | GND | FROM 7I77 |
| 4 | QB3 | TO 7I77 |
| 5 | /QB3 | TO 7I77 |
| 6 | +5V | FROM 7I77 |
| 7 | IDX3 | TO 7I77 |
| 8 | /IDX3 | TO 7I77 |
| 9 | QA4 | TO 7I77 |
| 10 | /QA4 | TO 7I77 |
| 11 | GND | FROM 7I77 |
| 12 | QB4 | TO 7I77 |
| 13 | /QB4 | TO 7I77 |
| 14 | +5V | FROM 7I77 |
| 15 | IDX4 | TO 7I77 |
| 16 | /IDX4 | TO 7I77 |
| 17 | QA5 | TO 7I77 |
| 18 | /QA5 | TO 7I77 |
| 19 | GND | FROM 7I77 |
| 20 | QB5 | TO 7I77 |
| 21 | /QB5 | TO 7I77 |
| 22 | +5V | FROM 7I77 |
| 23 | IDX5 | TO 7I77 |
| 24 | /IDX5 | TO 7I77 |

TB5 ANALOG DRIVE INTERFACE

TB5 is the 7I77s analog drive interface connector. It has both the +-10V analog outputs and the drive enable connections. Each drive interface uses 4 pins. TB5 is a 24 pin 3.5 MM pluggable terminal block supplied with six four pin removable screw terminal plugs.

| TB5 PIN | SIGNAL | DIRECTION |
| --- | --- | --- |
| 1 | ENA0- | FROM 7I77 |
| 2 | ENA0+ | FROM 7I77 |
| 3 | GND | FROM 7I77 |
| 4 | AOUT0 | FROM 7I77 |
| 5 | ENA1- | FROM 7I77 |
| 6 | ENA1 + | FROM 7I77 |
| 7 | GND | FROM 7I77 |
| 8 | AOUT1 | FROM 7I77 |
| 9 | ENA2- | FROM 7I77 |
| 10 | ENA2+ | FROM 7I77 |
| 11 | GND | FROM 7I77 |
| 12 | AOUT2 | FROM 7I77 |
| 13 | ENA3- | FROM 7I77 |
| 14 | ENA3+ | FROM 7I77 |
| 15 | GND | FROM 7I77 |
| 16 | AOUT3 | FROM 7I77 |
| 17 | ENA4- | FROM 7I77 |
| 18 | ENA4+ | FROM 7I77 |
| 19 | GND | FROM 7I77 |
| 20 | AOUT4 | FROM 7I77 |
| 21 | ENA5- | FROM 7I77 |
| 22 | ENA5+ | FROM 7I77 |
| 23 | GND | FROM 7I77 |
| 24 | AOUT5 | FROM 7I77 |

TB6 RS-422 INTERFACE

The 7I77 has a single RS-422 port (TB6) for I/O expansion. TB6 is a eight pin 3.5 MM pluggable terminal block supplied with one eight pin removable screw terminal plug.TB6 pinout is as follows:

| TB6 PIN | SIGNAL | DIRECTION |
| --- | --- | --- |
| 1 | GND | FROM 7I77 |
| 2 | GND | FROM 7I77 |
| 3 | RX0 | TO 7I77 |
| 4 | /RX0 | TO 7I77 |
| 5 | TX0 | FROM 7I77 |
| 6 | /TX0 | FROM 7I77 |
| 7 | +5V | FROM 7I77 |
| 8 | +5V | FROM 7I77 |

FIELD INPUT/OUTPUT CONNECTORS

Terminal blocks TB8 and TB7 are the 7I77s field input and output terminals. Inputs 0 through 15 and outputs 0 through 7 are terminated at TB8. Inputs 16 through 31 and outputs 8 through 15 are terminated at TB7. TB8 and TB7 are 3.5 MM pluggable terminal block with supplied removable screw terminal plugs. Pin one is at the bottom edge of the 7I77 card.

TB8 CONNECTOR PINOUT

| TB8 PIN | I/O | TB8 PIN | I/O |
| --- | --- | --- | --- |
| 1 | INPUT0 | 13 | INPUT12 |
| 2 | INPUT1 | 14 | INPUT13 |
| 3 | INPUT2 | 15 | INPUT14 |
| 4 | INPUT3 | 16 | INPUT15 |
| 5 | INPUT4 | 17 | OUTPUT0 |
| 6 | INPUT5 | 18 | OUTPUT1 |
| 7 | INPUT6 | 19 | OUTPUT2 |
| 8 | INPUT7 | 20 | OUTPUT3 |
| 9 | INPUT8 | 21 | OUTPUT4 |
| 10 | INPUT9 | 22 | OUTPUT5 |
| 11 | INPUT10 | 23 | OUTPUT6 |
| 12 | INPUT11 | 24 | OUTPUT7 |

FIELD INPUT/OUTPUT CONNECTORS TB7 CONNECTOR PINOUT

| TB7 PIN | OUTPUT | TB7 PIN | OUTPUT |
| --- | --- | --- | --- |
| 1 | INPUT16 | 13 | INPUT28 |
| 2 | INPUT17 | 14 | INPUT29 |
| 3 | INPUT18 | 15 | INPUT30 |
| 4 | INPUT19 | 16 | INPUT31 |
| 5 | INPUT20 | 17 | OUTPUT8 |
| 6 | INPUT21 | 18 | OUTPUT9 |
| 7 | INPUT22 | 19 | OUTPUT10 |
| 8 | INPUT23 | 20 | OUTPUT11 |
| 9 | INPUT24 | 21 | OUTPUT12 |
| 10 | INPUT25 | 22 | OUTPUT13 |
| 11 | INPUT26 | 23 | OUTPUT14 |
| 12 | INPUT27 | 24 | OUTPUT15 |

OPERATION

HOST INTERFACE

The 7I77 is intended to operate with a FPGA card with parallel port pinout like the Mesa 5I25 or 6I25. The FPGA card supports the step/dir, encoder, and smart serial interface used by the field I/O and spindle interface and expansion RS-422 port. The FPGA card can also supply 5V power to the 7I77.

EXTERNAL 5V POWER

The 7I77s 5V power for the analog, RS-422, and encoder section has an approximately 650 mA current drain. Installed encoders, RS-422 devices, and load on the analog outputs can increase this to up to 2A. This is in excess of the host FPGA cards current supply capacity. So when longer cables or high power drain encoders are used, or PC 5V is toward the low side, external 5V power should be supplied to the 7I77s TB1 connector. Both the FPGA card and the 7I77 have power jumpers that must be correctly set to match the 5V power source (see hardware configuration section)

STATUS LEDS

Six status LEDs are provided for power and field I/O operation monitoring. These LEDs are listed here:

| LED | COLOR | LOCATION | FUNCTION |
| --- | --- | --- | --- |
| CR1 | Yellow | Upper left | 5V monitor |
| CR3 | Yellow | Upper right | Field voltage monitor |
| CR6 | Yellow | Lower middle | +16V monitor |
| CR7 | Yellow | Lower middle | -12V monitor |
| CR15 | Green | Lower right | Field I/O activity |
| CR16 | Red | Lower right | Field I/O fault |

In normal operation CR1 through CR7 must always be on. At power-up, CR15 should be off and CR16 on. The red LED CR16 indicates a watchdog fault, which is expected before host communications are established. Once running, CR15 should blink at about 1 Hz for a 1 KHz update rate, and CR16 should be off. Note: CR15 and CR16 are only present in revision B or greater 7I77 cards.