Features:

Working  voltage ：A/B 5V Version : DC 3.7-5.5V; C 12V Version : DC 6-25V

Working  current：5-8MA

NO : no DS18B20 temperature sensor; TS : with DS18B20 temperature sensor

A: 5V version TTL232 interface;B: 5V version RS485 interface; C: 12V version RS485 interface

MODBUS RTU protocol，03 Read，06Write.

Serial baud rate：9600（default），N，8，1

By modifying the RS485 address, up to 247 module s can be connected (more than 16 US ether485 repeater)

Temperature sensor is DS18B20

Measures Temperatures from -55°C to +125°C(-67°F to +257°F)

±0.5°C Accuracy from -10°C to +85°C

Size：A/B Version 30X15X3.5mm; C Version 30X15X6mm

Weight：no DS18B20 1.4-1.9g,with DS18B20 1.9-2.2g

Use Please refer to : "R46CA01 modbus rtu protocol"

Wiring diagram: