

      CNCMASTERKIT             www.cncmasterkit.ru  www.чпуфорум.рф  www.мастерчпу.рф

TB6560HQV3-T3

3 AXIS DRIVER USER MANUAL
[image: image1.png]




CATALOGUE
1. Overview
2
32. TB6560AHQ advantag


32.1、At low speed operation System Advantages


42.2、In high-speed operation System Advantages


3. TB6560T3V1 Driver Board Introduction
4
54. TB6560T3V1connection drawing


5. The definition of parallel port pins
5
6. The connection of 4th axis 
7
7. The connection of limit switch
8
98. The setting of current,microstepping,decay mode


98.1、current setting


108.2、microstepping setting


118.3、decay mode setting


119. The connection of kinds of stepper motors


1310. The choice of stepper motor and power supply


1511. MACH software user manual


1511.1、Mach3 open


1611.2、Mach3 sofware setting


11.3、Limit switch’s mach3 setting
20
2111.4、Run of the G code



   
1. Overview
The PC Engraving is a Multifunction Engraving Machine collects engrave and Milling.The machine mainly applicable to Processing of a variety of mold like : Embossing plate,Shoe mold, Button mold, zipper-mode, Text die and stamping die design, equipment molds, glass molds and so on.The machine also applies to the advertising industry like: Division licenses, signs, architectural models, badges, badges, nameplates, panels, logo, numbers, signs, craft decoration, furniture decoration and so on. Besides, applies to portrait, landscape, calligraphy lettering, seals and other graphic art sculpture, YIN, YANG Wen-profile, relief production.

Our shop products 3-axis engraving machine drive,using high-performance dedicated micro-step away from the computer control chip TB6560, Open microcomputer control according to user requirements to functional design to the driver board, the composition of the minimum control system. he control panel is suitable for any small and medium-driven two-phase or four-phase hybrid stepper motor. And have current 0.6A, 1.2A, 1.8A, 2.5A 4 stalls adjustable function, support MACH2, MACH3 Series software, support KCAM4 Series software, extensive application and mold machining, engraving and other graphic applications. As a result of new bipolar constant-current chopping technique, high precision, the motor is running, little vibration, low noise, smooth operation, safe and convenient, the vast number of DIY enthusiasts and engraving machine manufacturers product of choice.

2. TB6560AHQ advantage
2.1  At low speed operation System Advantages

Low-speed operating system, is the clock frequency is not high, with small current drive-based systems, such as the speed of a few per minute to 100 turn, the user under the conditions in such applications, such as the use of traditional driving scheme, or a result of integrated chips segmentation is too low, leaving low-speed vibration is too large; or had to choose a high drive segments, so that the cost of an unnecessary increase.
TB6560AHQ driver chip's advantages:
(1). Motor vibration, low noise: Because the chip comes with an optional sub-2,4,16, enough to meet nearly every minute to switch from a few of the applications.

(2). embedded drive less heat: chip built-in cooling large enough alone to support small current-driven thermal requirements.

(3).support a variety of stepper motor selection: customers can choose a hybrid or a slightly larger torque permanent magnet stepper motor, the motor work in the allowed maximum torque of between 30-50%, the electrical costs of almost unchanged; chip provides multi-range current set-up and current-decay mode, supporting the same power index a variety of different parameters under the stepper motor.

2.2  In high-speed operation System Advantages

High-speed operating system refers to a higher clock frequency, and a large current drive-based systems, such as the speed of close to 1000 per minute to switch, under the conditions of such application-driven programs such as the use of traditional or integrated chip segment is too low due to , leaving the system speed range is too small; or because of the excessive breakdown of the high increase in cost, may also occur due to decline in high-frequency torque caused by vibration and noise.

TB6560AHQ driver chip's advantages:

(1). motor vibration, low noise: As the chip comes with 16 sub-TB6560AHQ chip functionality, to meet nearly every minute to switch from a few to the application requirements, and automatically generates a pure sine wave control current, and other high-integrated chip compared to the same high-speed torque is not only not fall under the contrary, increase; TB6560AHQ chip can withstand the peak due to the driving voltage of 40V, 3.5A peak current for the motor torque in the large, high-speed continuous operation under the offer of the technical support.

(2). supports a variety of stepper motor selection: customers can choose a hybrid moment slightly larger or permanent magnet stepper motor, so that the motor operates at maximum torque of between 30-50%, the electrical costs of almost unchanged; chip provides high-current set-up and multi-range current-decay mode, supporting the same power index a variety of different parameters under the stepper motor.
(3). an embedded easy to drive, small size cooling: large current drive, the chip surface to facilitate cooling radiator outside the company can also be directly connected to the user's original controller, metal shell, the embedded drives small size, easy to heat.

 In short, due to a high degree of TB6560AHQ chip, the external circuit is extremely simple, highly reliable, support NEMA23 and some NEMA34 stepper motor tens to nearly turn the wide speed range applications, will enable CNC equipment R & D costs and production costs both declined.
3. TB6560T3V1 Driver Board Introduction

The company introduced TB6560T3V1 is the company's accumulated years of design experience drives that the design made of 3-axis engraving machine drive. Main features except the 6560 itself, the other features are as follows:

1. it can drive three axis,The 4th axis expansion, to facilitate your needs 4-axis machining when working
2. Spindle relay output, easy to use software such as mach3 to control spindle start and stop.
3. Semi-flow control functions, and effective to stop the motor when the current is reduced to a minimum.
4. Interface with a fan, you can add fan under your choice.
5. With 3 wire 0.8-3.5A (peak) rated output of two-phase adjustable-current bipolar stepper motor drive;
6. Standard parallel port interface, support for MACH2, KCAM4 series software;
7. With optical isolation and DCDC power supply isolation, the full protection of your computer parallel port and equipment;
8. with limit switch,can connection 4 aixs limit switch
9. Support four microstepping ——1、1/2、1/4、1/16；

10. 12—36V single switching power supply,microchips to use as a 5V power supply, stable and heat small
11. Using RC +7414 automatic semi-flow,the current automatic reduce when stepper motor stop to down the heat
4. TB6560T3V1 Appearance

[image: image2.jpg]Relay port can control

A the spindie

Limit port can
connection 4 axds

12:36V DC input voltage

Parallel port

The th axis port

The mrosteppingand [ ran
Three axis current docay mode seting of mmse";’.'"r wo
seting of DIP swich ey oo e





Fig 1
5.  Interface and its definition
[image: image3.jpg]



Fig 2
25-pin parallel port control is defined as follows:
	DB25 control pin (PIN)
	The pin of the drive board
	Comment

	1
	EN
	All axes enable

	2
	STEPX
	X(First axis) pulse signal

	3
	DIRX
	X(First axis) direction of the signal

	4
	STEPY
	Y(Second axis)pulse signal

	5
	DIRY
	Y(Second axis)direction of the signal

	6
	STEPZ
	Z(Third axis) pulse signal

	7
	DIRZ
	Z（The third axis) direction of the signal

	10
	LIMIT-1
	Limit input interface 1

	11
	LIMIT-2
	Limit input interface 2

	12
	LIMIT-3
	Limit input interface 3

	13
	LIMIT-4
	Limit input interface 4

	14
	Relay
	

	15
	Blank
	

	16
	STEPB-
	B(4th axis) pulse signal

	17
	DIRB-
	B(4th axis) direction of the signal

	18-25
	GND
	


6.  The 4th aixs expansion port connection:
[image: image4.jpg]GND

EN ( Enable)
-CW(or )

PU/CK (step )
- 5V (Output)





Fig 3

7. The limit switch port connection:
[image: image5.jpg]VGG

The 4th axis limit switch
Z axis limit switch
Y axis limit switch
X axis limit switch





                           Fig 4
8. The setting of current,microstepping and current-decay mode adjustable
[image: image6.jpg]Microstepping setting M1/M2 Decay mode setting D1/D2




8.1、Current-decay mode setting
Pls use 12-24V,>4A switching power supply
Board marked D1D2 (match with DIP swtich 1 and 2) to set the value of switching current-decay
D1/D2： ON/ON——100%；   ON/OF——25%；   

                   OF/ON——50%；
  OF/OF——0%；

	D1
	D2
	Working mode

	ON
	ON
	Fast decay

	OF
	ON
	50% fast decay

	ON
	OF
	25%fast decay 

	OFF
	OFF
	Slow decay



[image: image7]

8.2、microstepping setting
Board marked M1M2(match with DIP swtich 3 and 4) to set the value of switching microstepping resolution
	M1
	M2
	Working mode

	OFF
	ON
	1/16

	OFF
	OFF
	1/4

	ON
	OFF
	1/2

	ON
	ON
	1


Remark:For motor run smoothly, please try to select high segments, such as 1 / 16 microstepping
8.3、current setting
[image: image8.jpg]



Board marked T1T2(match with DIP swtich 1 and 2) to set the value of switching current setting
	T1
	T2
	Current

	ON
	ON
	25%*2.5A

	OFF
	ON
	50%*2.5A

	ON
	OFF
	75%*2.5A

	OFF
	OFF
	100%*2.5A


Proposed current as close as possible stepper motor rated current

9. The connection of kinds of stepper motors
4 lead wires stepper motor:
[image: image9.png]



6 lead wires stepper motor:
[image: image10.png]



8 lead wires stepper motor:
[image: image11.png]



Stepper motor +A  -A  +B  -B connection AP AM BP BM of board port
10. Drive board and motor match:
The driver can drive two or four phase stepper motors,motor drive in order to obtain the most satisfactory results, need to select a reasonable set of supply voltage and current. The supply voltage level of the decision of the high-speed electrical performance, while the current settings determine the motor output torque.
(1) A supply voltage selected: 
 In general, the more high of the supply voltage, the more torque in high speed, and able to avoid missing step at high speed. On the other hand, the voltage is too high may damage the driver,also big shock in low speed 
 (2) output current settings: 
 For the same motor, the greater the current set value, the greater the motor output torque, but the high heat for motor and driver. Therefore, the general situation is that the current is set to supply long-term work machine when warm but not too hot when the values.

1)when 4 or 6 lead wires stepper motors in high-speed mode: the output current is set to equal or slightly less than the motor rated current value; 

 2) when 6 lead wires stepper motor in high torque mode: the output current is set to 70% of motor rated current 

 3) 8 -wire motor connected in series: the output current is set to 70% of motor rated current 
 4) 8 -wire motor parallel connection: the output current can be set to 1.4 times more than the motor rated current.

[image: image12.jpg]8 wire motor connection
8 wire motor in series
4 wire motor i parallel,good performance high torque in low

in high speed speed

6 wire motor in

high torque mods © iTe motor n high speed mode




Note: The current setting 15-30 minutes after you run motors, such as the motor temperature is too high, it should reduce the current settings.if after reducing the current value, the motor output torque is not enough, please improve the cooling conditions to ensure that motors and drives are not hot.
11. MACH3 software introduction:
11.1、Mach3 activate:
[image: image13.png]Session Profile

Current Profies
Create Profie

Deete Profie





Fig 5 open MACH3 
Shown in Fig5, open the MACH3 software, and then select mach3MILL then OK!
11.2、Mach3 software setting:
[image: image14.jpg]F4 Mach3 CNC Controll
Fle Corfig_Function Cfgs_iew Wizards _Operator_Plgin Control_Help

rogram Run AA_| MDI A2 | ToolPath Al | Offsets At | Settings Atts | _Diagnostics A.7_|  Mil->G15 G80 G17 G40 G20 GID G4 G54 G49 G99 G64 GI7

|| 00000 &%

e
ToGo "
med

—N—
o Load Wizards | _Last Wizard o Y jog
File:[No File Loaded Tmfl,m Mode | | Follow

EditG.Codg _Rewind CtlW | [Tool Information | | Feed Rate [ Spindle Speed

Recent File z enas

FRO % SRO %
Close G-Code_| Reverse Tool 0 = i 100 fire
Load G.Code Dia. +0.0000 -

_scknoe M | 71" 40.0000 9 & e 9 &

FRO 6.00.

Flood CirlE_ M Auto Tool Zeto RPM
[cvmode Remember | Retur Feedrate - -

6.00.

Unitsfin 0,00
Units/Rev 0.00

Elapsed 00:00:00

|_so0 onorr curans_|[]

Spindle Speed
0

History | Clear Status'.l Profilez{viach3Mmill





Fig 6 mach3 main interface
Open the MACH3 as Fig6，there are some common buttons,then let us set up it!
[image: image15.jpg][ Hach3 C er )

Fie | Corfig FunctionCfgs View Wieards Operstor Plugin Control Help

Motor Turing
SeerdCnfs
Syaembers
Homing/Limits
Todbah
S s
s
s,
Tt |
Cntarine o
Sonde s 7
o 25 +0.0000 ||&m

Save Settings.

EditG-Code | Rewind Ciw | FTool Information | Feed Rate \ Spindle Speed
] rmmaim] Tool pmnleER | w0 [
_Load G.Code | Dia. +0.0000 oa @ o @

Set Next Line | H +0.0000 FRO 5.00

Auto Tool Zero : RPM 0
Remember | Return Feedra(: - Siov 0

Elansed 00:00:00
ess Reset . u Jog ONOFF Ctrl-Al-J l
0.000

_G-Codes | M-Codes

Spindle Speed
0

Unitstvin 0,00
Units/Rev 0.00

| ——
History | Clear ‘Status'.! Profilez{viach3mill





Fig 7  mach3 setting menu
Shown in Fig7, open the CONFIG menu under the PORT & PIN menu.

[image: image16.jpg]Encoder/UEG' = | Spinile Setup | 1L Options |
oA TRt T ST s T

Fort 1 Fort 72 el fads
W Port Ensble [ Port Ensble 3
4 B ™ Max CL ods ensblad
[ rort [psete " port ™ Max NC-10 Yave Drive
Entry in Hex 0-8 Entey in Kex 0-9 Progran restart

[~ Pins 29 as inp

Festart if changed

™ SherLine 1/2 False nor s
. Kernel 5 1 ™ lodbus TaputQutpat Suppo 5
File:[NoFile Load] | ~ ook e matoom_cdsooore C sovoone T~ Bodbus Flucks Supportel Follow
Lo S ™ 1P Wodbus sppert

[~ Bvent Driven Serial Cor

I Serve Seriel Link Feedb: Epindle Speed
SRO %
pnae cwrs 1750

=
_we | _EAw | "

-

Remember| Return [FETE S0y 0

Note: Software must be rastarted and motors
Kernel spead is

| i 00:00:00 = '
rgency Mode Active. Ziibt | [ sogonorramana || [V 0.0 Eelnds:Speed
G-Codes | M-Codes |  +0.000] UnitsRev 0,00 o

| : |





Fig 8  frequency setting
Circle 1 is for setting the fundamental frequency, this parameter affect the motor rotation speed. Set up and select Circle 2, configuration pin definitions
[image: image17.png]D1 A12 | Tooren ake | onset | _Setogs | Dipnostes Az |

ort Sev ad e Slecticn s s | G sigae |

ot [t oo [ov e [sptomic gt





Fig 9 The pin setting of Dir,Step
[image: image18.jpg]Encoder/WFS 5 1 Spindle Setwp. 1 WL Opic
e TS e oo e S100

S e FrTn B
Diat < <
1<mn < <
a1
il x o o x
x o o x
x o o x
x o o x
x o o x
x o o x
x o o x
x o o x v

Bine2 -8, 1, 1 16, wnd 1 e sutpotpins Fo other pin

willIlgl

— —




Fig 10
Pls amend the software setting according to this board definition.then select the Output Signas as Fig10,1 is Enable and 14 is relay.
11.3、Limit switch mach3 setting：

Click “input signal”，setting as Fig11
[image: image19.png]Program Run Att4_| MDI Alf2 | ToolPathatta | Offsets alt5 | _Settings atte

Y MPG MODE ™
Mode e CAL

Encoder/MFG s | Spindle Setup 1wt options
Fort Setup and fri Selection | Motor Outputs @ | ot Signats

S Engiler | Port ¥ Pin Bumber | hetive Lov | Erilatad | Hotkey =
X < 1 i < * |
= x o o x x o
Homs x o o x x o
T o« 1 1 o« * o
x 0 0 4 x 0
x 0 0 4 x 0
o« 1 12 o« * o
x 0 0 4 x 0
x 0 x x 0
o« 1 13 o« *

Pins 10-13 uts. Only thess 5 pis 7 be

wE | me
"eed Hol A |
<Spc> Block Delete | |
Set Next Line
__SetNextLine | "1 optionaistop | '
Line  d Flood CtiE_ | -
_Run From Here | Dwell | [Cv mode |





Fig 11
11.4、G-Code run：
[image: image20.jpg]P

DI Al2 | ToolPath Alt4

Offsets Alts

Settings Alt6 |

Close Fe(s)
Ext

F60.000000

G0 ¥0.000000 Y0.000000 Z0.200000
M3

560000000

G43H5

GI ¥0.000000 Y0.000000 Z0.200000
G0 X1.179950 Y4.004260 Z0.200000
G1 X1.179950 Y4.004260 Z-0.100000

Edit G-Code |

Recent File
Close G-Code
Load G-Code

Set NextLine |

Rey

Single BLK AN |

Reverse Run |

Block Delete [

Zinhioit
_+0.000

+0.0000

_ Diagnostics A7_|  Mil->G15 GBO G17 G40 G20 GIN GI4 G54 G493 GII GE4 GI7

Radius
Correct

[Tool Information

Tool . 0 s

Dia. +0.0000
H +0.0000

Auto Tool Zero,
Remember | Return
Elapsed 00:00:01

|[_sos onorr curana_[]

Feed Rate
OverRidden

FRO %
100

L2 1)

FRO 6.00.
Feedrate
6.00.
0.00
0.00

Units/Min
Units/Rev

=Mach3Mill

[ Spindle Speed
Loz I *

9B

RPM 0
S-ov 0

Spindle Speed





Fig 12 Open G file
When all setting is ok,then open the G-code as Fig 12.
[image: image21.jpg]File:|No File Loaded

E:
Recent
Close G-Code

G-Code

o

SREO: [Boweae <]

E
i o
crostop

NestCirce.tap

& of B

Spindle Speed
SRO %
[somaecwrs 1550

Load G-Code THED: ot 4 [#Fe ] o @
O [ew =] Wil
SetNextLing I~ LEAATH ®
Line RPM 0
Run From Here Remember | Return EEE S-ov 0
= .00 00:00:00 — .
sReset ... Emergen Zmnit | |[ g onorrcurang[] [VSM 0.00 =pinde Sieet
G-Codes | M.Codes |  +0.000] UnitsRev ___0.00 o

=Mach3Mill |





Fig 13 Open the MACH3 with G-code self test.
[image: image22.png]Jogtods B

Recenrile St uxsan |

Clon GCod. Reverse tun_ || FRO|

LoadG.otn Ea
Bk tetee M

Semting = -

st coe M
14 00,0000

[_otmore cuvans 1]

)

oy | oo StatUSICCooERIN £SO

Profil





Fig 14
Open the G code, you can see the red RESET flashing, you can use mouse click the RESET to stop the flashing, then you can press circle 2 position CYCLESTART run. 
 Also if you need to manual, then you can press TAB key to open the manual control panel

[image: image23.png]File:forTosmes

Tool Information

Ranerun (| ToO! 0] e s
e e el (= ;

Femembe) Reum [ Button Jog

£21300:00:00.

=AY

oo conStatus—|pofil

v ww
. =

I Pevo—y





                                  Fig 15
Q: The stepper motor driver board the specific role of current-decay What is this?





A: The stepper motor is basically a breakdown of the way current subdivision law, according to the phase current sine wave to be tangent to the current point as subdivision points. In the phase current to reach sub-point control is necessary to control the current decay, otherwise it would be if there will be no way to accurately point overshoot stay in the breakdown of perspective. The motor at different speeds in different choices of decay mode. Fast decay at high speed, low speed when the slow decay. Slow decay at high speed there will be vibrations, high noise problem. Low selection of quick decay can lead to motor weakness and in severe cases closed position. Motor Control IC, the current decay is the target of H-bridge switch the control mode. Slow decay, when the high side pipes closed, fast decay time high or low side tubes are closed. Mixed decay is a first, a rapid decay and then a slow decay, mixing ratio of decay time is also a result of chip and power are not the same as











PAGE  
1

